Chlorination guide line for household level water treatment (Point of Use)

Safe Storage is crucial in order to prevent recontamination.

Narrow mouthed storage vessels are recommended, such as jerry cans, or containers with lids.

Recommendations:

- Drinking water should be stored safely.

- Stored water should be covered to prevent contamination and mosquito breeding.

- To prevent contamination, water should be accessed using a tap or clean scoop.

- Storage containers should be cleaned twice a week using soap.

- Any unsafely stored water should be treated prior to drinking.

Note – In temporary living areas, rainwater STILL needs treatment

Almost all dirty water benefits from being able to sit overnight to let the dirt settle out. If the clearer water is poured off, all treatment techniques work better, on the clearer water and it tastes better.

Chlorine related water treatment products are used to disinfect the drinking water at household level. These are also useful for emergency situations and camps.

Following are the chlorine products available in Myanmar currently.

- WaterGuard

  - Product description

0.6% Sodium Hypochlorite Solution (NaOCl) (It is weak chlorine solution)

Produced and distributed by Population Services International/Myanmar,

15/C, Aung Min Gaung Street, Kamaryut Township, Yangon.

For supplies please contact Dr. Mya Than Tun

Tel: +95-1-526173, 524166, Fax: +95-1-527668

E-mail: wguard@psimyanmar.org

(Regarding FAQ for Sodium Hypochlorite, please see at following site for CDC’s FAQ on SWS.)

http://www.cdc.gov/safewater/publications_pages/pubs_faq.htm

  - Product presentation

250 ML bottles – This is normal product size and meant for household level water treatment. Each bottle can be used to treat 1,000 liter of water. The dose is 5 ml of WaterGuard for 20L of water when the water is not too turbid. But during emergency period when the source of water is not safe and turbid, the dose should be double which means 10 ml of WaterGuard should be used for 20 L of water. (This is CDC’s recommendation for turbid water).

20 Liter container – This is produced to response emergency relief, especially intended for the Nargis cyclone relief. This product will be faced out in rehabilitation and recovery periods. The dose is 10 ml for 20 L of water since the source will not be safe after the storm. 20L of WaterGuard can treat 40,000 liters of water with double dose.

  - Advantages

        - Produced locally in Myanmar and has approval from Myanmar FDA.

        - It has been distributed in Myanmar for over 4 years.

        - PSI has been implementing household water treatment program in Ayeyarwaddy division for 2 years with support from UNICEF, most of people already knew what WaterGuard is.

        - Easy to use.

        - Readily available

        - Cost effective

        - Has written instructions in Myanmar langue.

        - CDC recommended NaOCl for Safe Water System Program.

WaterGuard Label with Instructions.

  - Disadvantage

        - Liquid form, there is danger of accidental drinking.

        - Bulky, difficult to transport.

        - Chlorine demand changes with the quality of water, difficult to fixed the dose.

        - 20L Containers are huge and not suitable for distributing household base.

        - The production of 250 ml bottles can cope with only 100,000 households every month.

  - Recommendation

        - Suitable for the time of emergency.

        - Even with collected rain water, there is danger of recontamination because of unsafe storage, therefore WaterGuard still should be used with the normal dose which is ½ table spoonful or 5 ml for 20 liters of rain water.

        - PSI is distributing 20L containers free to partners agencies especially for the camps and medical personnel, and should be replace with 250 ml bottles as soon as possible which is suitable for household based distribution.

        - PSI will provide TOT to local relief worker on uses and procedure of treating water with WaterGuard.

        - PSI will find opportunity to include 250 ml Bottle of WaterGuard in Hygiene kit or Household Kit with some partners agencies.

- Aquatabs

  - Product description

Sodium Dichloroisocyanurate (NaDCC)

This is product of Medentech Ltd., Whitemill Industrial Estate, Wexford, Ireland.

For Samples and supplies, contact Mr. Michael Gately

Tel: +353 53 60040, Fax: +353 53 41271

E-mail: mgately@medentech.com

UNICEF is also distributing this 8.5 mg tablets currently.

  - Product presentation

Although Aquatabs are available in range of sizes to suit the different circumstances found in an emergency, 8.5 mg tablets for 1 liter, 33 mg tablets for 5 liters and  67 mg tablets for 10 liters of water are available here in Myanmar right now. They come in 10 tablets strip.

  - Advantage

        - Joint FAO/WHO Expert Committee on Food Additives have approved NaDCC for routine use for drinking water.

        - Suitable for travelers and emergency situation

        - Well known product.

        - Easy to use.

        - Compact and easy to transport.

        - Widely used by many organizations.

  - Disadvantage

        - Current available product is one tablet can treat 1 liter or 5 liters or 10 liters of water therefore will need millions of tablets.

        - Small tablet which can be accidentally swallow by children or people.

        - Imported item.

  - Recommendation

        - Suitable for the time of emergency.

        - Should also be included in Hygiene Kit or Household Kit.

        - Need to distribute widely

        - Relief workers should also carry these tablets to get safe water for themselves.

- Pur Sachet

  - Product description

Powdered coagulant and chlorine: Fe2 (SO4)3; Fe (III); Ca(OCl)2

PuR uses coagulation, flocculation, sedimentation, cloth filtration and chlorination for treating water.

This is a product of Procter and Gamble Co.

  - Product presentation

It comes in a sachet; each sachet can treat 10 litres of water

Sachets are part of a larger kit for family use (two 10-12 L buckets with lids, cloth for filtration, scissors, elastic band, and stir rod)

  - Advantage

        - Can be used to treat highly turbid water

        - Treatment scheme includes a bucket with a lid for safe storage

        - Removes suspended material and colour. Results in a visual change in water quality (which increases usage)

        - Comes in sachet and easy to transport.

        - Also good to use in time of emergency as the source of water is unsafe and turbid.

        - The manufacturer is offering donations to relief efforts in Myanmar.

        - Widely promoted in disasters situation around the world.

        - Usage instructions available in Myanmar and Karen languages.

        - Through coagulation/filtration chlorine resistant Giardia can be removed

  - Disadvantage

        - One sachet can treat only 10 liters of water.

        - Need to assemble a kit for each family

        - Potential cost

        - Must be imported

  - Recommendations

        - Need to do TOT of local relief workers and community leaders for PUR usage and distribution

        - Relief workers can easily carry PUR sachets as treatment option for themselves.

- HTH

  - Product description

65% Calcium Hypochlorite

  - Product presentation

Powder form

It comes in 45 Kg drum.

  - Advantage

        - Can treat large quantity of water.

  - Disadvantage

        - Not suitable for household level water treatment.

        - Bleach powder is corrosive and can burn the skin.

        - Need skill to treat pond and wells.

  - Recommendations

        - Only authorized health personnel or skilled people should handle the bleach.

Point of Use Emergency Water Disinfection

| Method of Water Treatment | Hints on Using Technology Effectively | What is Treated | What is NOT Treated |
| --- | --- | --- | --- |
| Boiling | Bring water to rolling boil, for at least 1 minute, for each 1000 meters above sea level add 1 minute to boiling time. | 100% effective in killing protozoa, bacteria, viruses. | Does not inactivate spores, it does not remove any metals or turbidity |
| Chlorine Compounds (eg. Waterguard & Aquatabs) | Contact time should be 30 minutes. Follow usage instructions. | Inactivates bacteria and viruses, some protozoa. Note: Size of common pathogens Protozoa – 2 to 15 µm Bacteria – 0.2 to 10 µm Viruses – 0.004 to 0.1 µm | Not effective to inactivate Cryptosporidium Effectiveness is reduced in turbid waters Does not remove salt, heavy metals, hydrocarbons. |
| PuR | Follow the user instructions specific to product, usually treat 10 L per dosage. | Removes protozoa and bacteria, some metals and inactivates viruses. | It does not remove salt, heavy metals, hydrocarbons |
| Iodine 2% iodine solution 10% iodine solution Iodine tablet Iodated resin | Contact time should be 30 minutes after following the user instructions. Common forms of iodine and dosages: Iodine 2% - 5 drops in 1 L Iodine 10% - 10 drops in I L Iodine tablets – 1 or 2 in I L Iodine resins – follow user instructions | Inactivates most bacteria and viruses and some protozoa. Note: filtering water with carbon filter after disinfection will remove iodine taste Iodine is more effective in turbid waters than chlorine. | Not effective to inactivate Cryptosporidium, does not remove salt, heavy metals or hydrocarbons. Pregnant women should not use iodine to treat water if possible and average person should not use > 2 months for continuous water treatment. |
| Portable Filtering Devices Ceramic filters Carbon filters | Cartridge pore size will dictate the effectiveness of the unit; recommend absolute pore size of 1 µm minimum. Most ceramic portable cartridges are 0.2 – 0.3 µm and require post filtration disinfection to ensure safe water. | 1 µm pore size will remove protozoa such as Giardia and Cryptosporidia as well as some bacteria such as Campylobacter, but not all viruses which can be as small as 0.004 µm in size. | Some bacteria and viruses will not be removed, post filtration disinfection required to ensure safe potable water. |

References: Adapted from WHO